PRODUCT SAFETY DATA SHEET

The information given in this Data Sheet refers to the Liquid Hardener Component (Hardener) used in the products listed below by Trade Name.  No information is given on the fillers used in these systems as they are not Dangerous Substances.  All data refers to the Liquid, Wet or Uncured state.

Supplier		Coo-Var

Lockwood Street

Hull

HU2 OHN

Telephone No.	01482-328053

Fax No.		01482-219266

---------------------------------------------------------------------------------------------------------------------------

1.  SUBSTANCE IDENTIFICATION

FLOORPACK – HARDENER Q265

---------------------------------------------------------------------------------------------------------------------------

2.  CHEMICAL COMPOSITION

Composition  Cycloaliphatic amine adduct containing isophorone diamine.

Information on ingredients

The ingredients listed below have been associated with one or more of the listed immediate and/or delayed(*) health hazards.  Risk of damage and effects depends upon duration and level of exposure.

BEFORE USING OR HANDLING, READ AND UNDERSTAND THE MATERIAL SAFETY DATA SHEET.

CAS/Registry No.	Material description		% Content

100-51-6		*Benzyl alcohol			35-50

Physical hazard: Non-classifiable

Health hazard: Harmful (Xn)

R20  Harmful by inhalation

R22  Harmful if swallowed

OEL: None established.

2855-13-2		*Isophorone diamine		35-50

Physical hazard: Non-classifiable

Health hazard:  Corrosive (C)

Health hazard:  Harmful (Xn)

R21  Harmful in contact with skin

R22  Harmful if swallowed

R34  Causes burns

R43  May cause sensitisation by skin contact

R52  Harmful to aquatic organisms.

R53  May cause long-term adverse effects in the aquatic environment.

OEL: None established.

---------------------------------------------------------------------------------------------------------------------------

3.  HAZARDS IDENTIFICATION

Physical hazard		Non-classifiable

Health hazard		Corrosive (C)

R20/21/22  		Harmful by inhalation, in contact with skin and if swallowed.

R34           		Causes burns

R43			May cause sensitisation by skin contact.

R52/53  	Harmful to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

# Page 1 of 4

4.  FIRST AID MEASURES

Ingestion		If accidentally swallowed, give large quantities of water or milk to dilute 			the effects on the stomach.  Do not induce vomiting.  Seek immediate 				medical attention.

Inhalation		If inhaled, remove to fresh air.  If not breathing, give artificial respiration, 			preferably mouth-to-mouth.  Seek medical attention.

Skin contact		In case of contact, immediately flush with plenty of water for at least 15 			minutes.  Remove contaminated clothing.  Seek immediate medical 				attention.

Eye contact		Immediately flush eyes with plenty of water for at least 15 minutes.  Eyes 			should be held apart during irrigation to ensure water contact with entire 			surface of eyes and lids.  Seek immediate medical attention.

---------------------------------------------------------------------------------------------------------------------------

5.  FIRE FIGHTING MEASURES

Non-flammable at room temperature, but will burn.  In case of fire, use water spray, dry chemical, foam or carbon dioxide extinguishers.  Use water to keep fire-exposed containers cool.  For large fires, fire-fighters should wear full emergency protective equipment including self-contained breathing apparatus.

---------------------------------------------------------------------------------------------------------------------------

6.  ACCIDENTAL RELEASE MEASURES

Always wear appropriate protective clothing (Refer to section 8)

Large quantities  Contain and absorb with sand or sawdust and prevent from entering drains or watercourses; should this occur, the relevant Local Authority must be notified.

Small quantities  Soak up with absorbent material and remove to a chemical disposal area.

---------------------------------------------------------------------------------------------------------------------------

7.  HANDLING AND STORAGE

Handling  Handle in accordance with good industrial hygiene and safety practices.  Refer to section 8 for appropriate personal protection equipment.  Wash thoroughly after handling.

Inhalation		Avoid breathing vapour.  Use with adequate ventilation.

Skin contact		Do not get on skin or on clothing.

Eyes			Do not get in eyes.

Storage  Keep in tightly sealed containers and store in a cool, dry place, avoiding exposure to heat, light, moisture and air.

---------------------------------------------------------------------------------------------------------------------------

8.  EXPOSURE CONTROLS/PERSONAL PROTECTION

EXPOSURE CONTROLS

Engineering controls  The following exposure control techniques may be used to effectively minimise employee exposure: local exhaust ventilation, enclosed system design, process isolation and remote control in combination with appropriate use of personal protective equipment and prudent work practices.  These techniques may not necessarily address all issues pertaining to your operations.  We, therefore, recommend that you consult with experts of your choice to determine whether or not your programmes are adequate.

If airborne contaminants are generated when the material is heated or handled, sufficient ventilation in volume and air flow patterns should be provided to keep air contaminant concentrations below acceptable levels.  Refer to the exposure limits as given in section 2.

PERSONAL PROTECTION

Wear synthetic apron and boots if contact is likely.  Where air contaminants can exceed acceptable criteria, use approved respiratory protection equipment.  Respirators should be selected on the form and concentration of contaminants in air.  Use goggles and face shield if contact is likely.  Wear impervious gloves as required to prevent skin contact.

# Page 2 of 4

9.  PHYSICAL AND CHEMICAL PROPERTIES

Appearance		Low viscosity liquid

Colour			Colourless

Boiling point		> 1500C

Flammability		Combustible

Autoignition temperature	> 2000C

Explosive limits		Not determined

Oxidising properties	Not applicable

Vapour pressure		Low

Relative density		1.0 g/cm3 at 250C

Water solubility		Slightly miscible

Fat solubility		Not determined

Partition coefficient	Not determined

Viscosity		400 mPa.s at 250C

---------------------------------------------------------------------------------------------------------------------------

10.  STABILITY AND REACTIVITY

Normally considered stable and inert.

Hazardous polymerisation  Will not occur.

Materials to avoid  Oxidisers, acids.

Conditions to avoid  None known.

Thermal decomposition products may include  Oxides of carbon and nitrogen.

Other hazards  None known.

---------------------------------------------------------------------------------------------------------------------------

11.  TOXICOLOGICAL INFORMATION

Immediate hazards

Ingestion		Harmful if swallowed.  Burns to mucous membranes, oesophagus or gastro-			intestinal tract can result.

Inhalation		Harmful by inhalation.  Liquid or vapour can cause burns to nose, throat 			and lungs.

Skin			Harmful in contact with skin.  Causes burns.

Eyes			Causes burns.  Risk of serious damage to eyes.

100-51-6		*Benzyl alcohol

Can cause central nervous system depression.  Signs and symptoms 				may include headache, dizziness, nausea, vomiting, unconsciousness and 			even asphyxiation.

Delayed hazards

2855-13-2		*Isophorone diamine

May cause sensitisation by skin contact.

---------------------------------------------------------------------------------------------------------------------------

12.  ECOLOGICAL INFORMATION

Mobility			Not determined.

Degradability		Not determined.

Bioaccumulative potential	Not determined.

Aquatic toxicity		Toxicity to bacteria, algae and higher marine organisms not tested.

COD: Not determined.

---------------------------------------------------------------------------------------------------------------------------

13.  DISPOSAL CONSIDERATIONS

DO NOT reuse containers residual product without commercial cleaning.  All waste should be disposed of using a Registered Waste Carrier operating under the Environmental Protection Act (Duty of Care) Regulations 1992 (S.I. No. 2839)

---------------------------------------------------------------------------------------------------------------------------

# Page 3 of 4

14.  TRANSPORT INFORMATION

UK Supply Classification	Corrosive, contains

2855-13-2	Isophorone diamine

UK Carriage Classification  Corrosive substance

UK Conveyance Classification  Corrosive substance

UN Class		8

UN Packing group	III

UN Number		1760 corrosive liquid  nos

(Contains isophorone diamine)

ADR			8,66c

IMO/IMDG		Class 8, group III

IATA			Class 8, group III

HS Code Number		382490 95

---------------------------------------------------------------------------------------------------------------------------

15.  REGULATORY INFORMATION

Label information

UK Supply Classification	Corrosive

Contains   		Isophorone diamine

Hazard symbols		Harmful

Corrosive

UN No.			1760

Risk (R) and safety phrases (S)

R20/21/22		Harmful by inhalation, in contact with skin and if swallowed.

R34			Causes burns.

R43			May cause sensitisation by skin contact.

R52/53  	Harmful to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

S26			In case of contact with eyes, rinse immediately with plenty of 	water.

S28			After contact with skin, wash immediately with plenty of water and soap.

S36/39			Wear suitable protective clothing and eye/face protection.

S45			In case of accident or if you feel unwell, seek medical advice 					immediately.

S61			Avoid release to the environment.  Refer to special instructions.

---------------------------------------------------------------------------------------------------------------------------

16.  OTHER INFORMATION

The information contained herein is based on our present knowledge.  However, this shall not constitute a guarantee for any specific product features and shall not establish a legally valid constructual relationship.

---------------------------------------------------------------------------------------------------------------------------

DATE OF ISSUE  JANUARY 2004 – REV. 2				                Page 4 of 4

FLOORPACK  Q/265

PRODUCT SAFETY DATA SHEET

The information given in this Data Sheet refers to the Liquid Resin Component (Base) used in the products listed below by Trade Name.  Some of the systems are filled and no data is given on the fillers as they are not Dangerous Substances.  All data refers to the Liquid, Wet or Uncured state.

Supplier	Coo-Var

Lockwood Street

Hull

HU2 OHN

Telephone No.	01482-328053

Fax No.	`	01482-219266

---------------------------------------------------------------------------------------------------------------------------

1.  SUBSTANCE IDENTIFICATION

FLOORPACK   BASE  Q/265

---------------------------------------------------------------------------------------------------------------------------

2.  CHEMICAL COMPOSITION

Modified liquid Bisphenol A/F-Epichlorohydrin  epoxide resin.

Information on ingredients

The ingredients listed below have been associated with one or more of the listed immediate and/or

delayed (*) health hazards.  Risk of damage and effects depends upon duration and level of exposure.

BEFORE USING OR HANDLING, READ AND UNDERSTAND THE MATERIAL SAFETY DATA SHEET.

CAS/Registry No.	Material Description			% Content

25068-38-6		*4,4’-Isopropylidenediphenol-

epichlorohydrin copolymer		50-70

(Number average molecular weight less than 700)

Physical hazard  Non-classifiable

Health hazard    Irritant (Xi)

R36   Irritating to eyes

R38   Irritating to skin

R43   May cause sensitisation by skin contact

R51   Toxic to aquatic organisms

R53   May cause long-term effects in the aquatic environment

OEL:  None established

28064-14-4		*Phenol-formaldehyde polymer-

glycidyl ether				25-35

Physical hazard Non classifiable

Health hazard   Irritant (Xi)

R36   Irritating to eyes

R38   Irritating to skin

R43   May cause sensitisation by skin contact

R51   Toxic to aquatic organisms.

R53   May cause long-term adverse effects in the aquatic environment.

OEL:  None established.

Page 1 of 4

---------------------------------------------------------------------------------------------------------------------------

3.  HAZARDS IDENTIFICATION

Physical hazard		Non-classifiable

Health hazard		Irritant (Xi)

R36/38	Irritating to eyes and skin

R43	May cause sensitisation by skin contact

---------------------------------------------------------------------------------------------------------------------------

4.  FIRST-AID MEASURES

Ingestion	If accidentally swallowed, give large quantities of water or milk to dilute the 			effects on the stomach.  Do not induce vomiting.  Seek medical attention.

Inhalation	If inhalation causes adverse effects, remove to fresh air.  If problem persists, 			seek medical attention.

Skin contact	In case of contact, immediately flush with plenty of water.  Remove 				contaminated clothing.  In case of prolonged irritation, seek medical attention.

Eye contact	Immediately flush eyes with plenty of water for at least 15 minutes.  Eyelids 			should be held apart during irrigation to ensure water contact with entire 			surface of eyes and lids.  Seek immediate medical attention.

---------------------------------------------------------------------------------------------------------------------------

5.  FIRE-FIGHTING MEASURES

Non-flammable at room temperature, but will burn.  In case of fire, use water spray, dry chemical, foam or carbon dioxide extinguishers.  Use water to keep fire-exposed containers cool.  For large fires, fire-fighters should wear full emergency protective equipment including self-contained breathing apparatus.

---------------------------------------------------------------------------------------------------------------------------

6.  ACCIDENTAL RELEASE MEASURES

Always wear appropriate protective clothing (Refer to section 8)

Large quantities		Contain and absorb with sand or sawdust and prevent from entering drains or watercourses; should this occur, the relevant Local Authority must be notified.

Small quantities		Soak up with absorbent material and remove to a chemical disposal area.

---------------------------------------------------------------------------------------------------------------------------

7.  HANDLING AND STORAGE

Handling

Handle in accordance with good industrial hygiene and safety practices.  Refer to Section 8 for appropriate personal protection equipment.  Wash thoroughly after handling.

Inhalation		Avoid prolonged or repeated breathing of vapour.

Skin contact		Avoid contact with skin or clothing.

Eyes			Avoid contact with eyes.

Storage   Keep in tightly sealed containers and store in a cool, dry place, avoiding exposure to heat, light, moisture and air.

Page 2 of 4

8.  EXPOSURE CONTROLS/PERSONAL PROTECTION

EXPOSURE CONTROLS

Engineering controls  The following exposure control techniques may be used effectively minimise employee exposure: local exhaust ventilation, enclosed system design, process isolation and remote control in combination with appropriate use of personal protective equipment and prudent work practices.  These techniques may not necessarily address all issues pertaining to your operations.  We, therefore, recommend that you consult with experts of your choice to determine whether or not your programmes are adequate.

If airborne contaminants are generated when the materials are heated or handled, sufficient ventilation in volume and air flow patterns should be provided to keep air contaminant concentrations below acceptable levels.  Refer to Section 2 which details specific exposure limits where applicable.

PERSONAL PROTECTION

Where air contaminants can exceed acceptable criteria, use approved protection equipment.  Respirators should be selected based on the form and concentration of contaminants in air.  Use goggles if contact is likely.  Wear impervious gloves as required to prevent skin contact.

---------------------------------------------------------------------------------------------------------------------------

9.  PHYSICAL AND CHEMICAL PROPERTIES

Appearance		Pale yellow liquid, slightly sweet odour.

Boiling point		>1500C

Flammability		Combustible

Autoignition temp.	>2000C

Explosive limits		Not applicable

Oxidising properties	Not applicable

Vapour pressure		< 0.1 mbar at 200C

Relative density		1.12 g/cm3 at 250C

Water solubility		Not miscible

Fat solubility		Not determined

Partition coefficient	Not determined

Viscosity		0.90 Pa.s at 250C

--------------------------------------------------------------------------------------------------------------------------

10.  STABILITY AND REACTIVITY

Normally considered stable and inert.

Hazardous polymerisation	Will not occur

Materials to avoid	Mineral acids, bases, amines and mercaptans.

Conditions to avoid	Not known

Thermal decomposition products may include oxides of carbon.

Other hazards		Not known.

---------------------------------------------------------------------------------------------------------------------------

11.  TOXICOLOGICAL INFORMATION

IMMEDIATE HAZARDS

Ingestion		May be harmful if accidentally swallowed.  Irritation to mucous membranes, 		oesophagus or gastro-intestinal tract can result.

Inhalation		May cause irritation of nose, throat and lungs.

Skin			Irritating to skin.

Eyes			Irritating to eyes.

DELAYED HAZARDS

25068-38-6		*4,4’-Isopropylidenediphenol-epichlorohydrin copolymer

(Number average molecular weight less than 700)

May cause sensitisation by skin contact.

28064-14-4		*Phenol-Formaldehyde Polymer-Glycidyl Ether

May cause sensitisation by skin contact.

--------------------------------------------------------------------------------------------------------------------------

Page 3 of 4

12.  ECOLOGICAL INFORMATION

Mobility			Not determined.

Degradability		Not determined.

Bioaccumulative potential	Not determined.

Aquatic toxicity	Toxicity to bacteria, algae and higher marine organisms not tested.

COD: Not determined.

---------------------------------------------------------------------------------------------------------------------------

13.  DISPOSAL CONSIDERATIONS

DO NOT reuse containers containing residual product without commercial cleaning.

All waste should be disposed of using a Registered Water Carrier operating under the Environmental Protection Act (Duty of Care) Regulations 1992 (S.I. No. 2839)

---------------------------------------------------------------------------------------------------------------------------

14.  TRANSPORT INFORMATION

UK Supply Classification		Irritant, contains

25068-38-6			*4,4’-Isopropylidenediphenol-Epichlorohydrin Copolymer

(Number average molecular weight less than 700)

28064-14-4			*Phenol-Formaldehyde Polymer-Glycidyl Ether

UK Carriage Classification	Environmentally Hazardous Substance

UN Conveyance Classification	Environmentally Hazardous Substance

UN Class			9

Packaging Group			III

UN Number			3082 Environmentally Hazardous Substance  (contains epoxy resin)

ADR				Class 9,  Group III

IMO/IMDG			Class 9,  Group III

IATA				Class 9,  Group III

HS Code Number			390730 00

---------------------------------------------------------------------------------------------------------------------------

15.  REGULATORY INFORMATION

Label information

UK Supply Classification	Irritant

Contains    		*4,4’-Isopropylidenediphenol-Epichlorohydrin Copolymer

(Number average molecular weight less than 700)

*Phenol-Formaldehyde Polymer-Glycidyl Ether

UN Number		3082

Risk (R) and safety phrases (S)

R36/38			Irritating to eyes and skin.

R43			May cause sensitisation by skin contact.

S24			Avoid contact with skin.

S26			In case of contact with eyes, rinse immediately with plenty of water.

S36			Wear suitable protective clothing.

S61			Avoid release to the environment.  Refer to special instructions/safety data 			sheet.

---------------------------------------------------------------------------------------------------------------------------

16.  OTHER INFORMATION

The information contained herein is based on our recent present knowledge.  However, this shall not constitute a guarantee for any specific product features and shall not establish a legally valid contractual relationship.

---------------------------------------------------------------------------------------------------------------------------

DATE OF ISSUE  JANUARY 2004 – REV. 2                                                                  Page 4 of 4

FLOORPACK  Q/265

The information given on this technical data sheet is correct to the best of our knowledge.  Due to the policy of continuous improvement and modernisation to the product range some of the details may be subject to alteration without notice.  If in any doubt please contact COO-VAR TECHNICAL services on 01482-328053 .

The details shown are of a technical nature and are not designed as instructions for use.  For this information please read and understand details given on the containers and the relevant product brochures.

Should you require any information contact your stockist or Coo-Var directly.